REVISIONS REV #; ECN #; Description of Change; Engineer; Approved; Date. 1; NDI; New Document; EPF; SJS; 06/11/18.

Description

# System Master Controller Interface Design Description

Contents

1.0	INTRODUCTION AND OVERVIEW	4

1.1	Purpose	4

1.2	Responsibility	4

1.3	Definitions and Acronyms	4

1.4	References	4

1.5	Product Overview	4

2.0	Physical Layer	4

2.1	USB	4

2.2	UART 115200 N 8	4

2.3	Ethernet	5

3.0	Data Link Layer	5

3.1	Binary Remote Controller on Ethernet	6

3.2	Binary Command Interpreter on Serial port	6

3.3	Text Based Command Interpreter (Console)	7

4.0	Application Layer Protocol	8

4.1	Binary Command Interpreter on Ethernet	8

4.1.1	Error Command (0x000001)	9

4.1.2	ALTO Amp Commands	11

4.1.3	ALTOMultinet	21

4.2	Binary Command Interpreter on serial port	23

4.2.1	Do nothing (0)	23

4.2.2	Heart beat (1)	23

4.2.3	Get system information (2)	24

4.2.4	Get system information response (3)	24

4.2.5	Get status (4)	24

4.2.6	Get status response (5)	24

4.2.7	Set bypass enable (6)	26

4.2.8	Set bypass enable response (7)	27

4.2.9	Set bypass (8)	27

4.2.10	Set bypass response (9)	28

4.2.11	Toggle bypass (10)	28

4.2.12	Toggle bypass response (11)	29

4.2.13	Set system information (12)	29

4.2.14	Set system information response (13)	29

4.2.15	Get firmware version (14)	30

4.2.16	Get firmware version response (15)	30

4.2.17	Enter programming mode (16)	30

4.2.18	Toggle buttons (18)	30

4.2.19	Toggle buttons response (19)	31

4.3	Text Based Command Interpreter (Console)	31

4.3.1	Help and “?”	31

4.3.2	Contact	32

4.3.3	About	32

4.3.4	Cls	32

4.3.5	Reprogram	32

4.3.6	Status	33

4.3.7	Voltage	33

4.3.8	Temp	33

4.3.9	Reboot	34

# INTRODUCTION AND OVERVIEW

## Purpose

This document describes the protocol that is used for all system inter-board communication for the Alto line of Mini Relay Controller.

## Responsibility

The Design Engineer responsible for the product is responsible to keep this document accurate and make sure that all engineers working on various aspects of the product always have the latest copy to work from.

## Definitions and Acronyms

| CMS | Cabin Management System |
| --- | --- |
| CAN | Controller Area Network |
| MRB | Mini Relay Box |
| MRC | Mini Relay Controller = Same as MRB |

## References

Product Specification for System Master Controller, DWG 107830PS

## Product Overview

The ALTO Mini Relay Control is a small module that provides high current switch capabilities to the CSS keypads. It incorporates a CAN bus interface for allowing remote control feature.

# Physical Layer

The mini relay controller has two communication interfaces to interact with the user.

## USB

The Mini Relay Box USB controller operates as a full-speed or low-speed function controller during point-to-point communications with USB Host, Device, or OTG functions. The controller complies with the USB 2.0 standard, which includes SUSPEND and RESUME signaling. 16 endpoints including two hard-wired for control transfers (one endpoint for IN and one endpoint for OUT) plus 14 endpoints defined by firmware along with a dynamic sizable FIFO support multiple packet queueing. μDMA access to the FIFO allows minimal interference from system software. Software-controlled connect and disconnect allows flexibility during USB device start-up. The controller complies with OTG Standard's Session Request Protocol (SRP) and Host Negotiation Protocol (HNP).

## UART 115200 N 8

The TM4C123GH6PM controller includes eight Universal Asynchronous Receiver/Transmitter (UART) with the following features:

- Programmable baud-rate generator allowing speeds up to 5 Mbps for regular speed (divide by 16) and 10 Mbps for high speed (divide by 8)

- Separate 16x8 transmit (TX) and receive (RX) FIFOs to reduce CPU interrupt service loading

- Programmable FIFO length, including 1-byte deep operation providing conventional double-buffered interface

- FIFO trigger levels of 1/8, 1/4, 1/2, 3/4, and 7/8

- Standard asynchronous communication bits for start, stop, and parity

- Line-break generation and detection

- Fully programmable serial interface characteristics

  - 5, 6, 7, or 8 data bits

  - Even, odd, stick, or no-parity bit generation/detection

  - 1 or 2 stop bit generation

- IrDA serial-IR (SIR) encoder/decoder providing

- Support for communication with ISO 7816 smart cards

- Modem flow control (on UART1)

- EIA-485 9-bit support

- Standard FIFO-level and End-of-Transmission interrupts

- Efficient transfers using Micro Direct Memory Access Controller (μDMA)

  - Separate channels for transmit and receive

  - Receive single request asserted when data is in the FIFO; burst request asserted at programmed FIFO level

  - Transmit single request asserted when there is space in the FIFO; burst request asserted at programmed FIFO level

The Mini Relay Controller has connection to the UART0 through the programming port P2.

| Port P2 Pin | Function |
| --- | --- |
| 1 | U0TX |
| 2 | U0RX |

The UART0 is configured to work with the following parameters.

- Baud rate: 115200bps

- Parity bit: N

- Data bits: 8

## Ethernet

# Data Link Layer

The System Master controller implements different interfaces that runs simultaneously on different ports.

- Serial: UART, RS232, RS422, RS458 and USB

- Ethernet

- CAN

## Binary Remote Controller on Ethernet

The System Master Controller implements a TCP server on port 1002.

- KIND. Is a 4bytes length. Indicates the command kind. The value sent is an unsigned integer with the most significant bit set to 1. For example, a command number ID 3 will have a kind field value of 0x80000003 = 0x80000000 (MSB to 1) | 0x03 (command ID).

- PAYLOAD. Used to send data needed by the command. The length of the payload can be 0 or more. The length of the payload is set by the command kind.

typedef struct {

int kind;

char payload[];

}TCPBin_CMD_t;

## Binary Command Interpreter on Serial port

The binary command interpreter is used to send and receive data between the board and other devices. The packet frames sent can have different sizes to improve the use of the communication channel. The Binary Command Interpreter is implemented on the USB port.

The format of a general frame used for the communication is as shown in Figure 3.1. The length of the frame is variable and depends on the kind of packet sent.

Figure 3.1

- HEADER. Is a 4bytes length. Mark the beginning of the frame and has the value in ASCII “ALTO”.

- KIND. Is a 4bytes length. Indicates the command kind. The value sent is an unsigned integer with the most significant bit set to 1. For example, a command number ID 3 will have a kind field value of 0x80000003 = 0x80000000 (MSB to 1) + 0x03 (command ID).

- PAYLOAD. Used to send data needed by the command. The length of the payload can be 0 or more, and multiple of 4bytes. The length of the payload is set by the command kind.

- CRC. Is a 4bytes length. This field is used for error detection. The value stored in this field is the CRC operation of the KIND field plus the PAYLOAD field. The CRC is calculated using an operation on 32bits data. The operation could be ADD, XOR, CRC32-IEEE, CRC32C, etc. For the USB is used ADD operation due the USB adds an extra layer of safety. The transmitter calculates the CRC of the KIND and PAYLOAD and place the negate value on the CRC field. The receiver operates the CRC on the KIND, PAYLOAD and CRC field and if the result is 0 considers the packet was transmitted without error. As an example, let’s say the KIND field is 3 (not using the MSB for simplicity) and the payload is an integer 5. The transmitter will calculate the CRC = 3 + 5 = 8 and stores -8 on the CRC field. The receiver receive the packet and operates the CRC on the fields KIND, PAYLOAD and CRC. CRC = 3 + 5 + (-8) = 0.

- EOF or End Of Frame. Is a 4bytes length. Mark the end of the frame and has a value of “\r\n\r\n”

The command interpreter shall be able to receive and decode the frames received in different packets. Figure 3.2 shows a worst case scenarios where the frame is received in three spread packets that could be because the hardware FIFO or the external device divide the frame in small packets and sends it. In this example, before the frame starts, there is a no valid data that could be from a previous frame with corrupted end of frame. In this scenario, the algorithm is capturing the different packets and store it in a buffer. When it finds a pattern for an EOF it tries to find the beginning of the frame. Once it finds it removes the not valid data if there is, and extract the whole frame from the buffer and sends it to the decoder. Now the buffer is left with the rest of the data received.

Figure 3.2

The Figure 3.3 shows graphically the steps taken by the binary interpreter to find a frame and extract it.

Figure 3.3

## Text Based Command Interpreter (Console)

The text based command interpreter process a command line string into arguments and execute the first argument as a command.

The interpreter will take the supplied command line string and break it up into individual arguments.  The first argument is treated as a command and is searched for in the command table.  If the command is found, then the command function is called and all of the command line arguments are passed in the normal argc, argv form.

# Application Layer Protocol

This section describes the different kind of commands and presents the format used in the MRB.

## Binary Command Interpreter on Ethernet

The following table shows the ID for the different commands interfaces with the MRB.

| Command | Command ID |
| --- | --- |
| Do nothing | 0x00000000 |
| Error | 0x00000001 |
| ALTO Amp Get Input Select | 0x00000064 |
| ALTO Amp Get Input Select Response | 0x00000065 |
| ALTO Amp Set Input Select | 0x00000066 |
| ALTO Amp Set Input Select Response | 0x00000067 |
| ALTO Amp Get Volume | 0x00000068 |
| ALTO Amp Get Volume Response | 0x00000069 |
| ALTO Amp Set Volume | 0x0000006A |
| ALTO Amp Set Volume Response | 0x0000006B |
| ALTO Amp Get Bass | 0x0000006C |
| ALTO Amp Get Bass Response | 0x0000006D |
| ALTO Amp Set Bass | 0x0000006E |
| ALTO Amp Set Bass Response | 0x0000006F |
| ALTO Amp Get Treble | 0x00000070 |
| ALTO Amp Get Treble Response | 0x00000071 |
| ALTO Amp Set Treble | 0x00000072 |
| ALTO Amp Set Treble Response | 0x00000073 |
| ALTO Amp Get Mute | 0x00000074 |
| ALTO Amp Get Mute Response | 0x00000075 |
| ALTO Amp Set Mute | 0x00000076 |
| ALTO Amp Set Mute Response | 0x00000077 |
| ALTO Amp Get Compressor | 0x00000078 |
| ALTO Amp Get Compressor Response | 0x00000079 |
| ALTO Amp Set Compressor | 0x0000007A |
| ALTO Amp Set Compressor Response | 0x0000007B |
| ALTO Amp Get Loudness | 0x0000007C |
| ALTO Amp Get Loudness Response | 0x0000007D |
| ALTO Amp Set Loudness | 0x0000007E |
| ALTO Amp Set Loudness Response | 0x0000007F |
| ALTO Amp Get HP Input Select | 0x00000080 |
| ALTO Amp Get HP Input Select Response | 0x00000081 |
| ALTO Amp Set HP Input Select | 0x00000082 |
| ALTO Amp Set HP Input Select Response | 0x00000083 |
| ALTO Amp Get HP Volume | 0x00000084 |
| ALTO Amp Get HP Volume Response | 0x00000085 |
| ALTO Amp Set HP Volume | 0x00000086 |
| ALTO Amp Set HP Volume Response | 0x00000087 |
| ALTO Amp Get HP Mute | 0x00000088 |
| ALTO Amp Get HP Mute Response | 0x00000089 |
| ALTO Amp Set HP Mute | 0x0000008A |
| ALTO Amp Set HP Mute Response | 0x0000008B |
| ALTO Multinet Get Relay | 0x00000096 |
| ALTO Multinet Set Relay Response | 0x00000097 |
| ALTO Multinet Set Input | 0x000000A0 |
| ALTO Multinet Set Input Response | 0x000000A1 |
| ALTO Multinet Trigger Local Remote | 0x000000A2 |
| ALTO Multinet Get Gate | 0x000000A4 |
| ALTO Multinet Set Gate | 0x000000A6 |
| ALTO Multinet Get Pot | 0x000000B4 |
| ALTO Multinet Get Pot Response | 0x000000B5 |
| SMC System Get Monitor Status | 0x000000C8 |
| SMC System Get Monitor Status Response |  |
| SMC System Get Compiled Time |  |
| SMC System getCompiledTimeResponse |  |
| SMC System getRAMDeviceList |  |
| SMC System getRAMDeviceListResponse |  |
| SMC System getFlashDeviceList, |  |
| SMC System getFlashDeviceListResponse, |  |
| SMC System getFlashFileData, |  |
| SMC System getFlashFileDataResponse, |  |
| SMC System setFlashFileData, |  |
| SMC System setFlashFileDataResponse, |  |
| SMC System deleteFlashFileData, |  |
| SMC System deleteFlashFileDataResponse, |  |
| SMC System getManufacturerInformationData, |  |
| SMC System getManufacturerInformationDataResponse, |  |
| SMC System setManufacturerInformationData, |  |
| SMC System setManufacturerInformationDataResponse, |  |
| SMC System getConfigurationFile, |  |
| SMC System getConfigurationFileResponse, |  |
| SMC System setConfigurationFile, |  |
| SMC System setConfigurationFileResponse, |  |
| SMC FlashMM IFS getStatus | 0x00000122 |
| SMC FlashMM IFS getStatusResponse, |  |
| SMC FlashMM IFS format, |  |
| SMC FlashMM IFS formatResponse, |  |
| SMC FlashMM EFS getStatus, |  |
| SMC FlashMM EFS getStatusResponse, |  |
| SMC FlashMM EFS format, |  |
| SMC FlashMM EFS formatResponse, |  |
| SMC DiscreteIO get5VOutputPowerStatus | 0x00000140 |
| SMC DiscreteIO get5VOutputPowerStatusResponse, |  |
| SMC DiscreteIO turnOn5VOutputPower |  |
| SMC DiscreteIO turnOn5VOutputPowerResponse, |  |
| SMC DiscreteIO turnOff5VOutputPower, |  |
| SMC DiscreteIO turnOff5VOutputPowerResponse, |  |
| SMC DiscreteIO set5VOutputPower, |  |
| SMC DiscreteIO set5VOutputPowerResponse, |  |
| SMC DiscreteIO getDIOConfiguration |  |
| SMC DiscreteIO getDIOConfigurationResponse |  |
| SMC DiscreteIO setDIODirection |  |
| SMC DiscreteIO setDIODirectionResponse |  |
| SMC DiscreteIO setDIConfiguration |  |
| SMC DiscreteIO setDIConfigurationResponse |  |
| SMC DiscreteIO setDOConfiguration |  |
| SMC DiscreteIO setDOConfigurationResponse |  |
| SMC DiscreteIO setDOValue |  |
| SMC DiscreteIO setDOValueResponse |  |
| SMC DiscreteIO setDOToggle |  |
| SMC DiscreteIO setDOToggleResponse |  |
| Panasonic getMonitorStatus | 0x00000154 |
| Panasonic getMonitorStatusResponse |  |
| SMC DAC setVoltage | 0x0000015E |
| SMC DAC setVoltageResponse |  |
| SMC TEST resetCounters | 0x00000190 |
| SMC TEST resetCountersResponse |  |
| SMC TEST getCounters |  |
| SMC TEST getCountersResponse |  |
| SMC TEST setGPIOAsInput |  |
| SMC TEST setGPIOAsInputResponse |  |
| SMC TEST setGPIOAsOutput |  |
| SMC TEST setGPIOAsOutputResponse |  |
| SMC TEST setGPIOOutputValue |  |
| SMC TEST setGPIOOutputValueResponse |  |
| SMC TEST getGPIOInputValue |  |
| SMC TEST getGPIOInputValueResponse |  |
| SMC TEST MemoryTestEEPROM |  |
| SMC TEST MemoryTestEEPROMResponse |  |
| SMC TEST MemoryTestInternalFlash |  |
| SMC TEST MemoryTestInternalFlashResponse |  |
| SMC TEST MemoryTestExternalFlash |  |
| SMC TEST MemoryTestExternalFlashResponse |  |
| SMC TEST setIROutputValue, |  |
| SMC TEST setIROutputValueResponse, |  |
| SMC TEST getIRInputValue, |  |
| SMC TEST getIRInputValueResponse |  |
| ALTOAmp DirectCommand1 | 0x000001F4 |
| ALTOAmp DirectCommand1Response |  |
| ALTOAmp DirectCommand2 |  |
| ALTOAmp DirectCommand2Response |  |
| ALTOAmp getActiveConfigDatabase |  |
| ALTOAmp getActiveConfigDatabaseResponse |  |
| ALTOAmp setActiveConfigDatabase |  |
| ALTOAmp setActiveConfigDatabaseResponse |  |
| ALTOAmp getSoftChangeConfigDatabase |  |
| ALTOAmp getSoftChangeConfigDatabaseResponse |  |
| ALTOAmp setSoftChangeConfigDatabase |  |
| ALTOAmp setSoftChangeConfigDatabaseResponse |  |
| ALTOAmp getAnalogSelectAB 1 |  |
| ALTOAmp getAnalogSelectAB 1Response |  |
| ALTOAmp setAnalogSelectAB 1 |  |
| ALTOAmp setAnalogSelectAB 1Response |  |
| ALTOAmp getAnalogSelectAB 2 |  |
| ALTOAmp getAnalogSelectAB 2Response |  |
| ALTOAmp setAnalogSelectAB 2 |  |
| ALTOAmp setAnalogSelectAB 2Response |  |
| ALTOAmp getAudioFormat |  |
| ALTOAmp getAudioFormatResponse |  |
| ALTOAmp setAudioFormat |  |
| ALTOAmp setAudioFormatResponse |  |
| ALTOAmp getSurroundEnable |  |
| ALTOAmp getSurroundEnableResponse |  |
| ALTOAmp setSurroundEnable |  |
| ALTOAmp setSurroundEnableResponse |  |
| ALTOAmp getDSPRegisterRW |  |
| ALTOAmp getDSPRegisterRWResponse |  |
| ALTOAmp setDSPRegisterRW |  |
| ALTOAmp setDSPRegisterRWResponse |  |
| AVDS DirectCommand1 | 0x000002BC |
| AVDS DirectCommand1Response |  |
| AVDS DirectCommand2, |  |
| AVDS DirectCommand2Response, |  |
| ROSEN DirectCommand1 | 0x00000320 |
| ROSEN DirectCommand1Response |  |
| ROSEN DirectCommand2 |  |
| ROSEN DirectCommand2Response |  |
| ROSEN PowerSet |  |
| ROSEN PowerSetResponse |  |
| ROSEN PowerGet |  |
| ROSEN PowerGetResponse |  |
| ROSEN SourceSet |  |
| ROSEN SourceSetResponse |  |
| ROSEN SourceGet |  |
| ROSEN SourceGetResponse |  |
| ROSEN DVDControlSet |  |
| ROSEN DVDControlSetResponse |  |
| ROSEN DVDControlGet |  |
| ROSEN DVDControlGetResponse |  |
| ROSEN DVDLoadedGet |  |
| ROSEN DVDLoadedGetResponse |  |
| ROSEN DVDPlayingGet |  |
| ROSEN DVDPlayingGetResponse |  |

### Error Command

The error command is returned from the Remote Controller module in the SMC to the application to indicate an error produced inside the unit.

| Type | Device ID | Service UUID | Characteristic ID | Error Type | Error Payload |
| --- | --- | --- | --- | --- | --- |
| [4bytes]=0x80000001 | [4 bytes] | [2 bytes] | [2 bytes] | [4 bytes] | [Optional] |

- Type: Is always 0x80000001

- Device ID: Indicates the Device Id of the module that returned the error. If Error comes from the Remote Controller module the Device ID is 2.

- Service UUID: Indicates the services that generated the error inside the module.

- Characteristic ID: Indicates the characteristic that generated the error inside the Service.

There are multiples causes that can generate an error. The following table indicates the values and description of every possible error returned by the SMC. The first 4 error are generated by the Remote Controller Module and are independent on the module that handle the device itself.

| Error Type | Value | Description |
| --- | --- | --- |
| Command not implemented | 1 | Returned by the Binary Command interpreter on the Remote control module when the command send is not implemented |
| Command type out of Range | 2 | Returned by the Binary Command interpreter on the Remote control module when the command send is out of range |
| Command frame not recognized | 3 | Returned by the Binary Command interpreter on the Remote control module when the command frame doesn’t have the most significant bit set to 1 |
| Device Id not found | 4 | Returned by the Remote Control Module when can’t find a module with the Device id specified on the command sent to the SMC. The error payload will have 8 bytes with the following information. CmdType; Device Id. [4 bytes]; [4 bytes]. - CmdType: Indicates which command generated the error. - Device Id: Indicates the Device Id that the command CmdType was trying to reach. |
| ALTO Communication error | 5 | This error is generated by the ALTO Amp or ALTO Multinet devices and could it be by two reasons. - Transfer Failed: This error is generated when the transfer with an ALTO device failed. For example when the ALTO device don’t respond to a command and the timeout expires. - CRC error: This error is generated when the response to a command is received with CRC errors. The error payload field will be 4 bytes and indicates which caused this error. Error; Payload Value [4 bytes]. Transfer Failed; 0x00000001. CRC error; 0x00000002. |

### ALTO Amp Commands

#### ALTOAmpInputSelect

Input Select chooses the input source on amplifier with this function implemented for one or more zones. A bit field is set to determine the zones in which the selection is made. The amplifier may have an input active mask and if a non-active input is selected then an error will be returned.  Inc and Dec wrap around to active inputs.

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Zone Select | Bit field | 1 | Bit 0: Zone 1: 0 / 1 Bit 1: Zone 2: 0 / 1 | Set the zone bit(s) affected by the select |
| Input | Byte | 1 | 1 - 4 | Selects the input |

Get

| Type | Device ID |
| --- | --- |
| [4bytes]=0x80000064 | [4 bytes] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Zone1Val | Zone2Val |
| --- | --- | --- | --- | --- | --- |
| [4bytes]=0x80000065 | [4 bytes] | [2bytes]=0x0809 | [2bytes]=0x0001 | [1byte] | [1byte] |

Set

| Type | Device ID | Zone Select | Zone1Val | Zone2Val |
| --- | --- | --- | --- | --- |
| [4bytes]=0x80000066 | [4 bytes] | [1byte] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | ACKNAK |
| --- | --- | --- | --- | --- |
| [4bytes]=0x80000067 | [4 bytes] | [2bytes]=0x0809 | [2bytes]=0x0002 | [1byte] |

#### ALTOAmpVolume

Volume affects the entertainment volume for one or more zones. A bit field is set to determine the zones in which the volume is changed. Volume is a 32 position control which uses a pre-determined taper.

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Zone Select | Bit field | 1 | Bit 0: Zone 1: 0 / 1 Bit 1: Zone 2: 0 / 1 | Set the zone bit(s) affected by the volume |
| Zone N Volume | Byte | 1 | 0 - 31 | 0: mute, 31: maximum volume, N = Zone |

Get:

| Type | Device ID |
| --- | --- |
| [4bytes]=0x80000068 | [4 bytes] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Zone1Val | Zone2Val |
| --- | --- | --- | --- | --- | --- |
| [4bytes]=0x80000069 | [4 bytes] | [2bytes]=0x0811 | [2bytes]=0x0001 | [1byte] | [1byte] |

Set

| Type | Device ID | Zone Select | Zone1Val | Zone2Val |
| --- | --- | --- | --- | --- |
| [4bytes]=0x8000006A | [4 bytes] | [1byte] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | ACKNAK |
| --- | --- | --- | --- | --- |
| [4bytes]=0x8000006B | [4 bytes] | [2bytes]=0x0811 | [2bytes]=0x0002 | [1byte] |

#### ALTOAmpBass

Bass affects the entertainment bass for one or more zones. A bit field is set to determine the zones in which the bass is changed. Bass is a 15 position control. Center position is 0, maximum cut -7, maximum boost +7.  Cut or boost for each position is stored in a database using a pre-determined taper.

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Zone Select | Bit field | 1 | Bit 0: Zone 1: 0 / 1 Bit 1: Zone 2: 0 / 1 | Set the zone bit(s) affected by the bass |
| Zone N Bass | Signed | 1 | -7 to +7 | 0: center, -7: max cut, +7: max boost, N= Zone |

Get:

| Type | Device ID |
| --- | --- |
| [4bytes]=0x8000006C | [4 bytes] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Zone1Val | Zone2Val |
| --- | --- | --- | --- | --- | --- |
| [4bytes]=0x8000006D | [4 bytes] | [2bytes]=0x0812 | [2bytes]=0x0001 | [1byte] | [1byte] |

Set

| Type | Device ID | Zone Select | Zone1Val | Zone2Val |
| --- | --- | --- | --- | --- |
| [4bytes]=0x8000006E | [4 bytes] | [1byte] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | ACKNAK |
| --- | --- | --- | --- | --- |
| [4bytes]=0x8000006F | [4 bytes] | [2bytes]=0x0812 | [2bytes]=0x0002 | [1byte] |

#### ALTOAmpTreble

Treble affects the entertainment treble for one or more zones. A bit field is set to determine the zones in which the treble is changed. Treble is a 15 position control. Center position is 0, maximum cut -7, maximum boost +7.  Cut or boost for each position is stored in a database using a pre-determined taper.

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Zone Select | Bit field | 1 | Bit 0: Zone 1: 0 / 1 Bit 1: Zone 2: 0 / 1 | Set the zone bit(s) affected by the treble |
| Zone N Treble | Signed | 1 | -7 to +7 | 0: center, -7: max cut, +7: max boost, N = Zone |

Get:

| Type | Device ID |
| --- | --- |
| [4bytes]=0x80000070 | [4 bytes] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Zone1Val | Zone2Val |
| --- | --- | --- | --- | --- | --- |
| [4bytes]=0x80000071 | [4 bytes] | [2bytes]=0x0813 | [2bytes]=0x0001 | [1byte] | [1byte] |

Set

| Type | Device ID | Zone Select | Zone1Val | Zone2Val |
| --- | --- | --- | --- | --- |
| [4bytes]=0x80000072 | [4 bytes] | [1byte] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | ACKNAK |
| --- | --- | --- | --- | --- |
| [4bytes]=0x80000073 | [4 bytes] | [2bytes]=0x0813 | [2bytes]=0x0002 | [1byte] |

#### ALTOAmpMute

Mute attenuates entertainment audio completely (with a soft envelope so there is no pop sound) for one or more zones. A bit field is set to determine the zones in which mute takes effect.

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Zone Select | Bit field | 1 | Bit 0: Zone 1: 0 / 1 Bit 1: Zone 2: 0 / 1 | Set the zone bit(s) affected by the Mute |
| Zone N Mute | Enum | 1 | Bit 0: 0/1 | 0:demute, 1:mute |

Get:

| Type | Device ID |
| --- | --- |
| [4bytes]=0x80000074 | [4 bytes] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Zone1Val | Zone2Val |
| --- | --- | --- | --- | --- | --- |
| [4bytes]=0x80000075 | [4 bytes] | [2bytes]=0x0814 | [2bytes]=0x0001 | [1byte] | [1byte] |

Set

| Type | Device ID | Zone Select | Zone1Val | Zone2Val |
| --- | --- | --- | --- | --- |
| [4bytes]=0x80000076 | [4 bytes] | [1byte] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | ACKNAK |
| --- | --- | --- | --- | --- |
| [4bytes]=0x80000077 | [4 bytes] | [2bytes]=0x0814 | [2bytes]=0x0002 | [1byte] |

#### ALTOAmpCompressor

Compressor settings affect entertainment audio in one or more zones. Enabling the compressor limits the dynamic range of the incoming signal allowing loud and quiet passages to achieve a better balance in a noisy environment. A bit field is set to determine the zones in which the compression is enabled.

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Zone Select | Bit field | 1 | Bit 0: Zone 1: 0 / 1 Bit 1: Zone 2: 0 / 1 | Set the zone bit(s) affected by the compressor |
| Zone N Compressor | Byte | 1 | 0 = Disabled 1 = Enabled | Set Enable or Disable |

Get:

| Type | Device ID |
| --- | --- |
| [4bytes]=0x80000078 | [4 bytes] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Zone1Val | Zone2Val |
| --- | --- | --- | --- | --- | --- |
| [4bytes]=0x80000079 | [4 bytes] | [2bytes]=0x0820 | [2bytes]=0x0001 | [1byte] | [1byte] |

Set

| Type | Device ID | Zone Select | Zone1Val | Zone2Val |
| --- | --- | --- | --- | --- |
| [4bytes]=0x8000007A | [4 bytes] | [1byte] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | ACKNAK |
| --- | --- | --- | --- | --- |
| [4bytes]=0x8000007B | [4 bytes] | [2bytes]=0x0820 | [2bytes]=0x0002 | [1byte] |

#### ALTOAmpLoudness

Loudness settings affect entertainment audio in one or more zones based upon the Fletcher-Munson curves. A bit field is set to determine the zones in which the loudness settings are applied.  Both Bass and Treble depth can be set independently.

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Zone Select | Bit field | 1 | Bit 0: Zone 1: 0 / 1 Bit 1: Zone 2: 0 / 1 | Set the zone bit(s) |
| ZoneN Boost | Bit Mask | 1 | Bit 0:1 – Bass boost 00 = None 01 = 6 dB 10 = 9 dB 11 = 12 dB Bit 2:3 – Treb boost 00 = None 01 = 4 dB 10 = 6 dB 11 = 8 dB | Set Max amount of boost at low volume |

Get:

| Type | Device ID |
| --- | --- |
| [4bytes]=0x8000007C | [4 bytes] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Zone1Val | Zone2Val |
| --- | --- | --- | --- | --- | --- |
| [4bytes]=0x8000007D | [4 bytes] | [2bytes]=0x0821 | [2bytes]=0x0001 | [1byte] | [1byte] |

Set

| Type | Device ID | Zone Select | Zone1Val | Zone2Val |
| --- | --- | --- | --- | --- |
| [4bytes]=0x8000007E | [4 bytes] | [1byte] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | ACKNAK |
| --- | --- | --- | --- | --- |
| [4bytes]=0x8000007F | [4 bytes] | [2bytes]=0x0821 | [2bytes]=0x0002 | [1byte] |

#### ALTO Amp Headphone Input Select

HPInput Select chooses the input source on the headphone inputs. The amplifier may have an input active mask and if a non-active input is selected then an error will be returned.  Inc and Dec wrap around to any active inputs.

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Headphone# | Byte | 1 | 0 – 6 | Selects which headphone output (1-6). 0 is a global broadcast to all headsets |
| Input | Byte | 1 | 1 - 4 | Selects the input |

Get:

| Type | Device ID | Headphone# |
| --- | --- | --- |
| [4bytes]=0x80000080 | [4 bytes] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Headphone# | Input |
| --- | --- | --- | --- | --- | --- |
| [4bytes]=0x80000081 | [4 bytes] | [2bytes]=0x0709 | [2bytes]=0x0001 | [1byte] | [1byte] |

Set:

| Type | Device ID | Headphone# | Input |
| --- | --- | --- | --- |
| [4bytes]=0x80000082 | [4 bytes] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | ACKNAK |
| --- | --- | --- | --- | --- |
| [4bytes]=0x80000083 | [4 bytes] | [2bytes]=0x0709 | [2bytes]=0x0002 | [1byte] |

#### ALTO Amp Headphone Volume

HPVolume affects the volume for one or all headphone outputs.  Volume is a 32 position control which uses a pre-determined taper.

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Headphone# | Byte | 1 | 0 – 6 | Select the headphone output (1-6) or 0 for global broadcast to all |
| Volume | Byte | 1 | 0 - 31 | 0: mute, 31: maximum volume |

Get:

| Type | Device ID | Headphone# |
| --- | --- | --- |
| [4bytes]=0x80000084 | [4 bytes] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Headphone# | Volume |
| --- | --- | --- | --- | --- | --- |
| [4bytes]=0x80000085 | [4 bytes] | [2bytes]=0x0711 | [2bytes]=0x0001 | [1byte] | [1byte] |

Set:

| Type | Device ID | Headphone# | Volume |
| --- | --- | --- | --- |
| [4bytes]=0x80000086 | [4 bytes] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | ACKNAK |
| --- | --- | --- | --- | --- |
| [4bytes]=0x80000087 | [4 bytes] | [2bytes]=0x0711 | [2bytes]=0x0002 | [1byte] |

#### ALTO Amp Headphone Mute

HPMute affects the mute for one or all headphone outputs.

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Headphone# | Byte | 1 | 0 – 6 | Select the headphone output (1-6) or 0 for global broadcast to all |
| Mute | Byte | 1 | 0 - 1 | 0: unmute, 1: mute |

Get:

| Type | Device ID | Headphone# |
| --- | --- | --- |
| [4bytes]=0x80000088 | [4 bytes] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Headphone# | Mute |
| --- | --- | --- | --- | --- | --- |
| [4bytes]=0x80000089 | [4 bytes] | [2bytes]=0x0714 | [2bytes]=0x0001 | [1byte] | [1byte] |

Set:

| Type | Device ID | Headphone# | Mute |
| --- | --- | --- | --- |
| [4bytes]=0x8000008A | [4 bytes] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | ACKNAK |
| --- | --- | --- | --- | --- |
| [4bytes]=0x8000008B | [4 bytes] | [2bytes]=0x0714 | [2bytes]=0x0002 | [1byte] |

### ALTOMultinet

Supports the Remote passing of Virtual Input commands between units.  Set command sets the Virtual Input(s) on a unit.  Get receives routing and input state info for a target unit, and SetGet does both at once to improve speed.

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Input Specifier | Byte | 1 | Full | Bit 6:7 00 = Single unit, 11 = Wildcard Bits 1:5 specifes the input # 0-26 Bit 0: 0 = Off, 1 = On |
| ORT Routing Specifier | Byte | 2 | Full | Bit 15: Ignored Bit 14: 0=ignore, 1=Flush due to overflow Bit 11-13: Destination Type: 010 = Relay, 011 = Temp Bit 6-10: Destination Address (0-31) Bit 1-5: Destination Virtual Input (0-26) Bit 0: Input State 0 = Off, 1 = On |
| Relay# | Byte | 1 | 0-7 | Relay Number. First relay = 0, second Relay = 1… 8th Relay = 7 |
| Multinet Address | Byte | 1 | Full | Bit 0:4 Address 0-31 Bit 5:7 unit Type 1=Amp, 2=RelayBox, 3=TempBox |
| Relay State | Byte | 1 | 0-1 | 0 = Relay is not triggered 1 = Relay is triggered |
| Pot 1 Step | Byte | 1 | 0 – 255 | Current Step setting |
| Pot 2 Step | Byte | 1 | 0 – 255 | Current Step setting |
| Dac 1 Step | Byte | 1 | 0 – 255 | Current Step setting |
| Dac 2 Step | Byte | 1 | 0 – 255 | Current Step setting |
| Dac 3 Step | Byte | 1 | 0 – 255 | Current Step setting |

Get Relay:

| Type | Device ID | MultinetAddress | Relay# |
| --- | --- | --- | --- |
| [4bytes]=0x80000096 | [4 bytes] | [1 byte] | [1 byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Multinet Address | Relay# | RelayState |
| --- | --- | --- | --- | --- | --- | --- |
| [4bytes]=0x80000097 | [4 bytes] | [2bytes] = 0x0110 | [2bytes] = 0x0002 | [1byte] | [1 byte] | [1byte][bool] |

Set Input:

| Type | Device ID | MultinetAddress | Input Specifier |
| --- | --- | --- | --- |
| [4bytes]=0x800000A0 | [4 bytes] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Multinet Address | Input Specifier | ACKNAK |
| --- | --- | --- | --- | --- | --- | --- |
| [4bytes]=0x800000A1 | [4 bytes] | [2bytes]= 0x0B51 | [2bytes]= 0x0001 | [1byte] | [1byte] | [1byte] |

Trigger Local Remote:

| Type | Device ID | MultinetAddress | Input Specifier |
| --- | --- | --- | --- |
| [4bytes]=0x800000A2 | [4 bytes] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Multinet Address | Input Specifier | ACKNAK |
| --- | --- | --- | --- | --- | --- | --- |
| [4bytes]=0x800000A1 | [4 bytes] | [2bytes]= 0x0B51 | [2bytes]= 0x0002 | [1byte] | [1byte] | [1byte] |

Get Gate:

| Type | Device ID | MultinetAddress | Input Specifier |
| --- | --- | --- | --- |
| [4bytes]=0x800000A4 | [4 bytes] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Multinet Address | Input Specifier | GateState |
| --- | --- | --- | --- | --- | --- | --- |
| [4bytes]=0x800000A1 | [4 bytes] | [2bytes]= 0x0B51 | [2bytes]= 0x0003 | [1byte] | [1byte] | [1byte] |

Set Gate:

| Type | Device ID | MultinetAddress | Input Specifier |
| --- | --- | --- | --- |
| [4bytes]=0x800000A6 | [4 bytes] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Multinet Address | Input Specifier | ACKNAK |
| --- | --- | --- | --- | --- | --- | --- |
| [4bytes]=0x800000A1 | [4 bytes] | [2bytes]= 0x0B51 | [2bytes]= 0x0004 | [1byte] | [1byte] | [1byte] |

Get Pot:

| Type | Device ID | MultinetAddress |
| --- | --- | --- |
| [4bytes]=0x800000B4 | [4 bytes] | [1 byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Multinet Address | Pot 1Step | Pot 2Step | Dac 1Step |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [4bytes]=0x800000B5 | [4 bytes] | [2bytes] = 0x0110 | [2bytes] = 0x0003 | [1byte] | [1 byte] | [1byte] | [1byte] |

| Dac 2Step | Dac 3Step |
| --- | --- |
| [1byte] | [1byte] |

### Rosen RS485 Commands

The Rosen commands control the Rosen devices. On the RS485 bus every device has a network ID.

| Command | Type | Value |
| --- | --- | --- |
| Power Set | 0x80000324 | 0x0f = ON, 0x10 = OFF, 0x3C = TOGLE |
| Power Get | 0x80000326 | 0x55 = For all |
| Power Get Response | 0x80000327 | 0x01 = For Power ON, 0x00 for Power OFF. See ping command in Rosen doc, need to extract from Byte 3, bit 4 |
| Source Set | 0x80000328 | 0x01 = Comp1, 0x02 = Comp2, 0x02 = S-Vid, 0x04 = ARGB, 0x12=ARGB2, 0x05=CMPN1, 0x14=CPMN2, 0x06=DVI1, 0x13=DVI2, 0x07=SDI1, 0x08=SDI2, 0x15=AUTOSDI, 0x10=Next, 0x11=Next-Scale |
| Source Get | 0x8000032A | 0x55 = For all |
| Source Get Response | 0x8000032B | see above, extract from Byte3 bits 0-3 |
| DVD Control Set | 0x8000032C | 0x01=Enter, 0x02=Left, 0x03=Right, 0x04=Up, 0x05=Down, 0x06=Eject, 0x07=Stop, 0x08=Next, 0x09=Prev, 0x0A=Subtitle, 0x0E=FFwd, 0x0F=Reverse, 0x10=Play, 0x11=PPToggle, 0x13=Menu, 0x20=Power |
| DVD Power Get | 0x8000032E | 0x01 = For all |
| DVD Power Get Response | 0x8000032F | power status extracted from byte2 bit 6 |
| DVD Loaded Get | 0x80000330 | 0x01 = For all |
| DVD Loaded Get Response | 0x80000331 | power status extracted from byte2 bit 0 |
| DVD Playing Get | 0x80000332 | 0x01 = For all |
| DVD Playing Get Response | 0x80000333 | power status extracted from byte2 bit 1 |

Set/Get:

| Type | Device ID | Network ID | Value |
| --- | --- | --- | --- |
| [4bytes]=0x80000XXX | [4 bytes] | [1byte] | [1byte] |

Response:

| Type | Device ID | Service UUID | Characteristic ID | Network ID | Value |
| --- | --- | --- | --- | --- | --- |
| [4bytes]=0x80000XXX | [4 bytes] | [2bytes] | [2bytes] | [1byte] | [1byte] |

## Binary Command Interpreter on serial port

The following table shows the ID for the different commands interfaces with the MRB.

| Command | Command ID |
| --- | --- |
| Do nothing | 0 |
| Heart beat | 1 |
| Get system Information | 2 |
| Get system Information Response | 3 |
| Get status | 4 |
| Get status response | 5 |
| Set bypass enable | 6 |
| Set bypass enable response | 7 |
| Set bypass | 8 |
| Set bypass response | 9 |
| Toggle bypass | 10 |
| Toggle bypass response | 11 |
| Set system information | 12 |
| Set system information response | 13 |
| Get firmware version | 14 |
| Get firmware version response | 15 |
| Enter programming mode | 16 |
| Toggle buttons | 18 |
| Toggle buttons response | 19 |

### Do nothing (0)

This command is ignored by the controller. The Kind id is 0 and has no payload. It can be send in both directions.

### Heart beat (1)

The heartbeat command is used to see if the device is alive and can receive commands. The heartbeat command responds with another heartbeat command. The kind ID is 1 and has no payload. It can be send in both directions.

### Get system information (2)

The Get System Information command is used to get the manufacturer information from the board. It has no payload and the ID 2. This command is sent from the host to the MRB and it responds with a Get System Information Response command (4.2.4) with ID 3.

### Get system information response (3)

This command is the response to a Get System Information command (ID 2). It contains the manufacturer information stored on the MRB. The payload field is 32 bytes wide and contains the following fields.

- Model Number. The first 16 bytes received after the Kind field. It contains a string of character with the model number printed in ASCII.

- Serial Number. 15 bytes formatted in ASCII with the device serial Number.

- # of Channels. 1 byte with the number of channels active.

### Get status (4)

This command is sent from the host to the MRB to ask for the status information of the board. It has no payload and the ID 4. The MRB responses with a Get Status Response command (ID 5).

### Get status response (5)

This command provide information of the internal state of the controller. It returns the input voltage, the USB voltage, the temperature measure in degree Celsius and for every channel it returns the mode (master/slave), function (momentary/toggle) and all the discrete inputs and outputs.

The following table shows a description of the different parameters on the payload.

| Bit/Field | Name | Description |
| --- | --- | --- |
| 0:3 bytes | Input Voltage | [Float] Input voltage Value |
| 4:7 bytes | USB Voltage | [Float] USB Input voltage Value |
| 8:9 bytes | Temperature C | [Int16] Temperature in degree Celsius |
| 10:11 bytes | Channel 0 | Status for Input 0 Bit/Field; Name; Description. 1; Master/Slave0; Master = 0, Slave = 1. 2; Momentary/Toggle0; Toggle = 0, Momentary = 1. 3:7; -; Not used. 8; Button0; Pressed = 1, not pressed = 0. 9; Indicator0; Active = 1, not active = 0. 10; Toggle State0; Active = 1, not active = 0. 11; Relay0; Active = 1, not active = 0. 12; Slave0; Active = 1, not active = 0. 13:15; -; Not used. |
| 12:13 bytes | Channel 1 | Status for Input 1 Bit/Field; Name; Description. 1; Master/Slave1; Master = 0, Slave = 1. 2; Momentary/Toggle1; Toggle = 0, Momentary = 1. 3:7; -; Not used. 8; Button1; Pressed = 1, not pressed = 0. 9; Indicator1; Active = 1, not active = 0. 10; Toggle State1; Active = 1, not active = 0. 11; Relay1; Active = 1, not active = 0. 12; Slave1; Active = 1, not active = 0. 13:15; -; Not used. |
| 14:15 bytes | Channel 2 | Status for Input 2 Bit/Field; Name; Description. 1; Master/Slave2; Master = 0, Slave = 1. 2; Momentary/Toggle2; Toggle = 0, Momentary = 1. 3:7; -; Not used. 8; Button2; Pressed = 1, not pressed = 0. 9; Indicator2; Active = 1, not active = 0. 10; Toggle State2; Active = 1, not active = 0. 11; Relay2; Active = 1, not active = 0. 12; Slave2; Active = 1, not active = 0. 13:15; -; Not used. |

### Set bypass enable (6)

This command enables the bypass option on the board. When a parameter is bypassed, the value is controlled with the Set Bypass Command (ID = 8). The payload field is 32 bits wide and contains the following fields.

- Address. 2 bytes as UInt16 data. Address for the different parameters that can be bypassed. The address map for the different parameters is presented on the next table.

- Value. 2 bytes. The value the bypass enable function takes. 1 for Enable, and 0 to disable.

The following table shows a description of the different parameters on the payload.

| Bit/Field | Name | Description |
| --- | --- | --- |
| 0:1 bytes | Value | [Int16] Enable bypass = 1, Disable bypass = 0 |
| 2:3 bytes | Address | [Uint16] Address of the parameter Parameter; Address. SW0; 0. SW1; 1. SW2; 2. IND0; 3. IND1; 4. IND2; 5. DIP_MT0; 6. DIP_MT1; 7. DIP_MT2; 8. DIP_MT3; 9. DIP_MS0; 10. DIP_MS1; 11. DIP_MS2; 12. DIP_MS3; 13. RELAY0; 14. RELAY1; 15. RELAY2; 16. SLAVE0; 17. SLAVE1; 18. SLAVE2; 19. User LED0; 20. User LED1; 21. User LED2; 22. |

### Set bypass enable response (7)

This is the response to the Set Bypass Enable command. It returns the same address as the Set Bypass Command with the actual bypass value after set it.

### Set bypass (8)

This command sets the bypass value on the board. A parameter is bypassed with the Set Bypass Enable command (ID = 6), and this command only works when the bypass Enable is set. The payload field is 32 bits wide and contains the following fields.

- Address. 2 bytes as UInt16 data. Address for the different parameters that can be bypassed. The address map for the different parameters is presented on the next table.

- Value. 2 bytes. The value the bypass function takes.

The following table shows a description of the different parameters on the payload.

| Bit/Field | Name | Description |
| --- | --- | --- |
| 0:1 bytes | Value | [Int16] Bypass value |
| 2:3 bytes | Address | [Uint16] Address of the parameter Parameter; Address. SW0; 0. SW1; 1. SW2; 2. IND0; 3. IND1; 4. IND2; 5. DIP_MT0; 6. DIP_MT1; 7. DIP_MT2; 8. DIP_MT3; 9. DIP_MS0; 10. DIP_MS1; 11. DIP_MS2; 12. DIP_MS3; 13. RELAY0; 14. RELAY1; 15. RELAY2; 16. SLAVE0; 17. SLAVE1; 18. SLAVE2; 19. User LED0; 20. User LED1; 21. User LED2; 22. |

### Set bypass response (9)

This is the response to the Set Bypass command (ID = 8). It returns the same address as the Set Bypass Command with the actual bypass value after set it.

### Toggle bypass (10)

This command toggles the bypass value on the board. A parameter is bypassed with the Set Bypass Enable command (ID = 6), and this command only works when the bypass Enable is set. The payload field is 32 bits wide and contains the following field.

- Address. 2 bytes as UInt16 data. Address for the different parameters that can be bypassed. The address map for the different parameters is presented on the next table.

The following table shows a description of the different parameters on the payload.

| Bit/Field | Name | Description |
| --- | --- | --- |
| 0:1 bytes | Value | Not used |
| 2:3 bytes | Address | [Uint16] Address of the parameter Parameter; Address. SW0; 0. SW1; 1. SW2; 2. IND0; 3. IND1; 4. IND2; 5. DIP_MT0; 6. DIP_MT1; 7. DIP_MT2; 8. DIP_MT3; 9. DIP_MS0; 10. DIP_MS1; 11. DIP_MS2; 12. DIP_MS3; 13. RELAY0; 14. RELAY1; 15. RELAY2; 16. SLAVE0; 17. SLAVE1; 18. SLAVE2; 19. User LED0; 20. User LED1; 21. User LED2; 22. |

### Toggle bypass response (11)

This is the response to the Toggle Bypass command (ID = 10). It returns the same address as the Toggle Bypass Command with the actual bypass value after set it.

### Set system information (12)

This command sets the manufacturer information. The manufacturer information is stored on the EEPROM inside the microcontroller. The payload field is 32 bytes wide and contains the following field.

- Model Number. The first 16 bytes received after the Kind field. It contains a string of character with the model number printed in ASCII.

- Serial Number. 15 bytes formatted in ASCII with the device serial Number.

- # of Channels. 1 byte with the number of channels active.

### Set system information response (13)

This command is the response to the Set System Information command. Once the manufacturer information is set, the information stored on the EEPROM is read and send it back with this command.

### Get firmware version (14)

This command request to the MRB to return the firmware version.

### Get firmware version response (15)

This command is the response to the Get Firmware Version. The payload is 4bytes wide and contains the firmware version in following format:

Major.Minor.Month.Build

### Enter programming mode (16)

This command tells the MRB to enter in Programming mode. This mode allows to update the firmware on the microcontroller through USB. This command doesn’t return any data.

### Toggle buttons (18)

This command is used to toggle the input value of the buttons. This command has no effect when the channel is configured as momentary or slave. The payload field is 32 bits wide and contains the following field.

- Address. 2 bytes as UInt16 data. Address for the different parameters that can be toggle. The address map for the different parameters is presented on the next table.

| Bit/Field | Name | Description |
| --- | --- | --- |
| 0:1 bytes | Value | Not used |
| 2:3 bytes | Address | [Uint16] Address of the parameter Parameter; Address. SW0; 0. SW1; 1. SW2; 2. |

### Toggle buttons response (19)

This commands is the response to the Toggle Buttons command. It includes the address for the button and the actual updated value of the button.

## Text Based Command Interpreter (Console)

Figure 4.1 shows the interface for the Text base command interface. This interface is accessed through the P2 port on the board using a UART with 3.3V.

Figure 4.1

When the unit starts up, it prints some logging information and then prints a prompt. After that the user can start typing commands. The interpreter allows to

- Write a command and multiples arguments in the same line.

- Correct the command written while typing.

Using the “help” command without arguments prints the list of commands the unit can accept and a quick description.

### Help and “?”

The help command without arguments prints a list of available commands as shown in Figure 4.2. When the argument is one of the commands, it prints the description for that particular command as shown in Figure 4.3.

| Figure 4.2 | Figure 4.3 |
| --- | --- |

### Contact

The contact command shows information about the ALTO company.

Figure 4.4

### About

The about command shows information about the product as well the number of channels it was configured.

Figure 4.5

### Cls

This command will clear the screen.

### Reprogram

This command the board to enter in programming mode. This mode allows to load a new firmware using the LM Flash Programmer.

Figure 4.6

### Status

This command display continuously information about the different channels. To exit from this command press the key “q”.

It start presenting information about the mode and function selected for each channel, then present the status information for each channel every 100ms.

- sw# is the state of the input switch button. 0 represents not pressed, 1 is pressed.

- is# is the internal state for the toggle function.

- ind# is the input indicator line state.

- R# is the status of the relay output line.

- S# is the status of the slave output line.

Figure 4.7

### Voltage

This command display continuously information about the input voltage and the USB voltage. To exit from this command press the key “q”.

The input voltage is shown in green when the voltage is above 26V, yellow when is between 21V and 26V, and red below 21V.

The USB input voltage is green when is above 4.3V and red when below.

Figure 4.8

### Temp

This command display continuously information about the temperature of the microcontroller. To exit from this command press the key “q”.

The temperature is shown in green when the temperature is below 35 degree, yellow when is between 35 and 60, and red above 60.

Figure 4.9

### Reboot

This command reset the board.

Figure 4.10