| REVISIONS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| REV # | ECN # | Description of Change | Engineer | Approved | Date |
| 1 | NDI | Initial Release | SJS | DFH | 9/20/16 |

| Doc Level | Responsible | Date |
| --- | --- | --- |
| 3 | Drafter |  |
|  | SJS | 9/10/2016 |
| 2 | Engineer |  |
|  | SJS | 9/10/2016 |
| 1 | Release to Production |  |
|  | DFH | 9/20/2016 |

Relay Controller

Interface Design Description

PRELIMINARY

1 Introduction	3

2 References	3

3 Physical Layer	3

4 Application Layer Protocol	4

4.1 Message Format	4

4.2 Classes	5

4.3 Operations	5

4.4 Functions	6

4.4.1 Diagnostic Class 0x01 Functions	6

4.4.1.1 DeviceDetailedStatus (Class 0x01, Function 0x10)	7

4.4.1.2 ConfigDataParameter (Class 0x01, Function 0x40)	9

4.4.1.3 ManufacturingInformation (Class 0x01, Function 0x70)	11

4.4.1.4 PrepareToRestart (Class 0x01, Function 0x09)	12

4.4.1.5 DiagRestart (Class 0x01, Function 0x63)	12

4.4.2  Clu Class 0x0B Functions	14

4.4.2.1 CluInpInfo (Class 0x0B, Function 0x10)	14

4.4.2.2 CluInpCfg (Class 0x0B, Function 0x11)	16

4.4.2.3 CluInpOverride (Class 0x0B, Function 0x12)	18

4.4.2.4 CluRlyIndOverride (Class 0x0B, Function 0x20)	20

4.4.2.5 CluPowerMaskCfg (Class 0x0B, Function 0x30	21

4.4.2.6 Clu5V28VCfg (Class 0x0B, Function 0x31)	22

4.4.2.7 CluTaskState (Class 0x0B, Function 0x32)	23

4.4.2.8 CluMasterOff (Class 0x0B, Function 0x33)	24

5 Appendix	25

5.1 AltoNET™ Interface Details	25

5.1.1 Physical Layer Details	25

5.1.2 Message Format	25

5.2 Configuration Table Editing	26

# Introduction

This document describes the protocol that is used for all system inter-board communication for the Alto line of Digital Audio amplifiers.

Data Dictionary

BCC		Block Check Character

CLU		Configurable Logic Unit

Clu		Configurable Logic Unit

Cfig		Configuration

Config		Configuration

Config Table	Configuration Table

Configuration	A collection of user settable parameters

DOC		Document

IDD		Interface Design Description

Ind		Clu Indicators

Inp		Clu Inputs

Ovr		Override

PRS		Product Requirements Specification

SRS		Software Requirements Specification

RC		Relay Controller

Rly		Clu Relays

RlyInd		Clu Relays and Indicators

# References

######XX		AltoNet Overview

######CT 		Relay Controller  Configuration Table Definitions

# Physical Layer

See Appendix Sections for Details regarding the Physical Layers.

# Application Layer Protocol

## Message Format

The external message format may vary depending upon the physical interface used.  See the Appendix section for details regarding the message format.

The Internal System Message format is common to all physical interfaces, and it is described as follows (See Appendix for any Prefix or Suffix added to the External Message):

| Class | Oper | Func | Seq | Length | Data Payload | BCC |
| --- | --- | --- | --- | --- | --- | --- |
| 8 bits | 8 bits | 8 bits | 8 bits | 8 bits | <Data 1>…<Data 26> | 8 bits |

- The first five bytes of the message are the header which are mandatory

- Data payload is optional so a data  length of ZERO is valid

- Unused payload bytes beyond the last byte of valid data are functionally ignored.

- Sequence Identifier is a numeric value that uniquely identifies a data transport protocol. It is a rolling number starting from 0 through 255. In the case of a parsing error or a missing number in the Seq series; the generated error message echoes the Seq of the offending message. When the Host is transmitting a message, it should increment the Seq for the next message. When the Client receives a message it will respond with the same Seq. If a message is missing, the Host may resend the message based on the importance of the message.

- Client responds with an AckNak message, for every “Set” command and a Response message for every “Get” command.

- There is a BCC-8 byte appended to the end of message.

  - The Message Acknowledge system returns the following data in the Ack/Nak byte of all messag0x00  (0)  Good Status

  - 0x40  Busy, Waiting to Respond

  - 0xFF (-1) Invalid BCC

  - 0xFE (-2) Invalid Argument

  - 0xFD (-3) Function Not Executed

  - 0xFC (-4) Invalid CRC

  - 0xFB (-5)  - 0xF0 (-15) N/A

  - 0xDF – 0xD4 N/A

  - 0xD4 Hardware Fault

  - 0xD3 – 0xCE N/A

  - 0xCD  Diag Session Not Active

  - 0xCC – 0xB8 N/A

  - 0xB7 – 0xB0 assigned to Relay Controller

  - 0xAF – 0x80 N/A

- The Message Acknowledge system returns the following data in the Seq byte of all messages:

  - Next Sequence Number for Ack

  - Echo the ID received for Nak

- Data (Endianess)

  - By convention, all scalar data types are presented in big Endian format.

  - In bit fields, bits are numbered from right to left:

| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

  - In Byte fields, bytes are number from right to left:

| Byte 1 | Byte 0 |
| --- | --- |

- Errors: In the event of errors, for example, Invalid Argument, Function Not Executed and Invalid BCC, the Error message is returned using the same sequence ID as the offending message.

- BCC is calculated as a running XOR of each byte in the message, excluding the prefix (32 bytes).

## Classes

Table of possible Class IDs is shown below:

Refer to ######XX AltoNet Overview for the definitive list of Classes

| Class ID | Class Name |
| --- | --- |
| 0x00 | N/A |
| 0x01 | Diagnostic |
| 0x02 | N/A |
| 0x03 | N/A |
| 0x04 | N/A |
| 0x08 | N/A |
| 0x09 | N/A |
| 0x0B | Clu |

## Operations

Table of possible Operation IDs is shown below:

| Operation ID | Name | Description |
| --- | --- | --- |
| 0x00 | Set | Host Writes data |
| 0x01 | Get | Host Reads data |
| 0x05 | Unsolicited | N/A RC Unsolocited |
| 0x06 | Response | Data returned by Client from a Get Operation |
| 0x07 | ACKNAK | Response to a message that was received |
| 0x08 | Status | N/A RC lStatus like heartbeat |
| 0x80 | MethodStart | Start a Method like DiagRestart |
| 0x82 | MethodAction | N/A RC Action like load, step, pause,continue &c |
| 0x84 | MethodStop | N/A RC Stop a Method |
| 0x86 | MethodResult | N/A RC Result of a Method |

## Functions

Each of the following sections lists all the possible Function tokens associated with a particular Class.

### Diagnostic Class 0x01 Functions

The Diagnostic Class provides an interface to monitor and manage devices in the system.

| Function ID | Function Name |
| --- | --- |
| 0x09 | PrepareForRestart |
| 0x10 | DeviceDetailedStatus |
| 0x20 | N/A |
| 0x40 | ConfigDataParameter |
| 0x63 | DiagRestart |
| 0x70 | ManufacturingInformation |
| 0x74 | N/A |
| 0x75 | N/A |
| 0x78 | N/A |

      - DeviceDetailedStatus (Class 0x01, Function 0x10)

If the Overall byte is ZERO then initialization is complete and there are no faults. If the Fault bit is set then examine the Fault byte for details. Note that the three 28V power services are AND-ed together to determine a fault.

The Config Table/EEPROM fault indicates that either the Config Table is invalid or an EEPROM Read or Write failed. Reloading the Config Table should correct the problem otherwise there is an EEPROM hardware fault.

The six voltages are unsigned scaled integer shorts with a resolution of 0.01 volts. The normal range of the dual supply Power 28B5V to the Clu Inputs depends upon the selection in the Config Table.

Both the 8 relay and 8 indicator ON/OFF states are adjusted for the three 28V power supply masks in the Config Table. In addition, the states are also adjusted for overrides via applicable Clu Class messages described later in this document. The state of the algorithm output is logical but the adjusted state is the actual physical.

Note that there are 27 Clu inputs thus the fourth of these bytes contains only three inputs. Because the input states are adjusted for overrides their logical state represented in this message may not be the phsical pin state. The logical state is what us used by the control algorithms.

| Name | Data Type | Size Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Overall | Bit field | 1 | Bit 0: 0 – 1 Bit 1: 0 – 1 | Fault: 0:normal, 1:fault Init: 0:normal, 1:initializing |
| Fault | Bit | 1 | Bit 0: 0 – 1 Bit 1: 0 – 1 Bit 2: 0 – 1 Bit 3: 0 – 1 Bit 4: 0 – 1 | 1 = fault 28V (AND of the 3 28V power services) 1 = fault 5V 1 = fault 8V 1 = fault 28V5V (dual power supply for Clu inputs) 1 = fault Config Table / EEPROM |
| Power 28V5V | Short 0.01V | 2 | 0 – 2800 + overrange 0 – 500 +overrange | Dual Supply selected in Config Table |
| Power5V | Short 0.01V | 2 | 0 – 500 + overrange | 5V Supply |
| Power 8V | Short 0.01V | 2 | 0 – 800 + overrange | 8V Supply |
| Power 28V 1 | Short 0.01V | 2 | 0 – 2800 + overrange | 1st of three 28V Services |
| Power 28V 2 | Short 0.01V | 2 | 0 – 2800 + overrange | 2nd of three 28V Services |
| Power 28V 3 | Short 0.01V | 2 | 0 – 2800 + overrange | 3rd of three 28V Services |
| Relay Results | Bit field | 1 | 8 one bit fields | Physical state 1=ON, 0=OFF of the 8 relays after inhibits and overrides |
| Indicator Results | Bit field | 1 | 8 one bit fields | Physical state 1=ON, 0=OFF of the 8 indicators after adjustment for inhibits and overrides |
| Input 0 – 7 Results | Bit field | 1 | 8 one bit fields | Logical state 1=ON, 0=OFF of 8 inputs after adjustment for overrides. May not be Pin State. |
| Input 8– 15 Results | Bit field | 1 | 8 one bit fields | Logical state 1=ON, 0=OFF of 8 inputs after adjustment for overrides. May not be Pin State. |
| Input 16 – 23 Results | Bit field | 1 | 8 one bit fields | Logical state 1=ON, 0=OFF of 8 inputs after adjustment for overrides. May not be Pin State. |
| Input 24– 26 Results | Bit field | 1 | 3 one bit fields | Logical state 1=ON, 0=OFF of 3 inputs after adjustment for overrides. May not be Pin State. |
| Input 0 – 7 Pin State | Bit field | 1 | 8 one bit fields | Logical state 1=ON, 0=OFF of 8 input pins after adjustment for overrides. May not be Pin State. |
| Input 8– 15 Pin State | Bit field | 1 | 8 one bit fields | Logical state 1=ON, 0=OFF of 8 inputs pins after adjustment for overrides. May not be Pin State. |
| Input 16 – 23 Pin State | Bit field | 1 | 8 one bit fields | Logical state 1=ON, 0=OFF of 8 inputs pins after adjustment for overrides. May not be Pin State. |
| Input 24– 26 Pin State | Bit field | 1 | 3 one bit fields | Logical state 1=ON, 0=OFF of 3 inputs pins after adjustment for overrides. May not be Pin State. |

Table of Detailed Status Data Fields

Operations Sent: Get, Unsolicited

DeviceDetailedStatus Get (Host to Client):

| Class | Oper | Func | Seq | Length | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x01 | 0x10 | N | 0 | 26 bytes x 0x00 | 0xyy |

DeviceDetailedStatus Unsolicited (Client to Host):

| Class | Oper | Func | Seq | Length | Data 1 | Data 2 | Data 3 | Data 4 | Data 5 | Data 6 | Data 7 | Data 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x05 | 0x10 | N | 24 | Overall | Fault | 28V5V | 28V5V | 5V | 5V | 8V | 8V |

| Data 9 | Data 10 | Data 11 | Data 12 | Data 13 | Data 14 | Data 15 | Data 16 | Data 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28V 1 | 28V 1 | 28V 2 | 28V 2 | 28V 3 | 28V 3 | Rly | Ind | Inp 0 -7 |

| Data 18 | Data 19 | Data 20 | Data 21 | Data 22 | Data 23 | Data 24 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inp 8 - 15 | Inp 16 - 23 | Inp 24 - 26 | Pin 0 - 7 | Pin 6 - 15 | Pin 16 - 23 | Pin 24 - 26 | 2 bytes x 0x00 | 0xyy |

Operations Returned: Response, AckNak

DeviceDetailedStatus Response from Get (Client to Host):

| Class | Oper | Func | Seq | Length | Data 1 | Data 2 | Data 3 | Data 4 | Data 5 | Data 6 | Data 7 | Data 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x06 | 0x10 | N | 24 | Overall | Fault | 28V5V | 28V5V | 5V | 5V | 8V | 8V |

| Data 9 | Data 10 | Data 11 | Data 12 | Data 13 | Data 14 | Data 15 | Data 16 | Data 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28V 1 | 28V 1 | 28V 2 | 28V 2 | 28V 3 | 28V 3 | Rly | Ind | Inp 0 -7 |

| Data 18 | Data 19 | Data 20 | Data 21 | Data 22 | Data 23 | Data 24 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inp 8 - 15 | Inp 16 - 23 | Inp 24 - 26 | Pin 0 - 7 | Pin 6 - 15 | Pin 16 - 23 | Pin 24 - 26 | 2 bytes x 0x00 | 0xyy |

DeviceDetailedStatus AckNak (Client to Host):

| Class | Oper | Func | Seq | Length | Data 1 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x07 | 0x10 | N | 1 | Ack Nak | 25 bytes x 0x00 | 0xyy |

DeviceDetailedStatus AckNak (Host to Client):

      - ConfigDataParameter (Class 0x01, Function 0x40)

This command dynamically updates the Configuration Parameter Values stored both in RAM and permanently in EEPROM. This command also retrieves other Configuration data like preset factory limits, data types, descriptions, etc., so that a list may be generated, viewed and/or printed.  Refer to Alto Dwg  ######CT, XXXXXX Configuration Table Definitions.xls for details pertaining to configuration table contents and how to interpret them. Parameters that are multiple bytes require special handling in software. For example, a 32-bit integer would require four Config Table lines. The Parameter Values, Limits for range check and Default Values would be assembled in the endian order required to be useful. Only single byte Parameters are handled in a general purpose manner.

The Relay Controller (RC) itself uses the actual Parameter (Value) and also the high / low limits and whether the Data Type is signed or not to range check the incoming Parameter Value. Some of tthe message and what is stored in the Config Table is ignored by the RC.

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Board | Enum | 1 | Ignored | Ignored by RC |
| Index | Byte | 1 | 0 - 255 | Index of the Parameter Cell |
| Multiple | Byte |  | Ignored | Ignored by RC |
| Parameter | Byte | 1 | 0 - 255 | Parameter Value being set |
| ASCII description | Byte (ASCII) | 20 | Valid ASCII values | Up to 20 character description of the data field. Ignored by RC. |
| Value | Byte | 1 | 0 - 255 | The current value being used for the Parameter |
| Low limit | Byte | 1 | 0 - 255 | The factory lower limit for the parameter |
| High limit | Byte | 1 | 0 - 255 | The factory higher limit for the parameter |
| Default | Byte | 1 | 0 - 255 | The factory default value |
| DataType | Byte | 1 | 0 - 255 | 0 -255 except 3 result in an unsigned range limit check. The value 3 results in a signed range limit check. |

Table of ConfigDataParameter Data Fields

Operations Sent: Get, Set

ConfigDataParameter Get (Host to Client):

| Class | Oper | Func | Seq | Length | Data 1 | Data 2 | Data 3 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x01 | 0x40 | N | 3 | Board | Index | Multiple | 23 bytes x 0x00 | 0xyy |

ConfigDataParameter Set (Host to Client): (Weight on Wheels active, sets Active Config values)

| Class | Oper | Func | Seq | Length | Data 1 | Data 2 | Data 3 | Data 4 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x00 | 0x40 | N | 4 | Board | Index | Multiple | Parameter | 22 bytes x 0x00 | 0xyy |

Operations Returned: Response, AckNak

ConfigDataParameter Response from Get (Client to Host):

| Class | Oper | Func | Seq | Length | Data 1 | Data 2 | Data 3 | Data 4 | Data 5 | Data 6 | Data 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x06 | 0x40 | N | 26 | Index | ASCII | ASCII | ASCII | ASCII | ASCII | ASCII |

| Data 8 | Data 9 | Data 10 | Data 11 | Data 12 | Data 13 | Data 14 | Data 15 | Data 16 | Data 17 | Data 18 | Data 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII | ASCII | ASCII | ASCII | ASCII | ASCII | ASCII | ASCII | ASCII | ASCII | ASCII | ASCII |

| Data 20 | Data 21 | Data 22 | Data 23 | Data 24 | Data 25 | Data 26 | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII | ASCII | Value | Low | High | Default | Datatype | 0xyy |

ConfigDataParameter AckNak:

| Class | Oper | Func | Seq | Length | Data 1 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x07 | 0x40 | N | 1 | AckNak | 25 bytes x 0x00 | 0xyy |

      - ManufacturingInformation (Class 0x01, Function 0x70)

This Code Version is embedded in Program Flash and the other Manufacturing Information (MI) is located in Serial EEPROM. The MI contains information about Software and Hardware, such as SW Code version,  HW version and HW Serial Number.  The Major and Minor version byte value can be 00 – 99.  Reporting of version information should always be padded to 2 digit  (ex: 07.02 not 7.2). There are 11 Bytes in both the Set and [Get] Response] reserved for use by manufacturing.

Note that location of this information in the messages matches the Amplifier. This leaves interleaved unused Data bytes.

Data Fields

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| SW Version | Byte | 2 | (00-99) . (00-99) | Software Version (major.minor) |
| HW Version | Byte | 1 | 00-255 | Hardware Version - The integer value must match the Hardware BOM version |
| Reserved1 | Byte | 1 | 00-255 | Reserved for Alto use |
| Unused | Byte | 1 | 00-255 | Returns 0 |
| Reserved2 | Byte | 1 | 00-255 | Reserved for Alto use |
| Unused | Byte | 2 | 00-255 | Return 0 |
| Reserved3 | Byte | 1 | 00-255 | Reserved for Alto use |
| Unused | Byte | 1 | 00-255 | Returns 0 |
| Unit Serial Number | Unsigned Long | 4 | 100000-999999 | Unit serial number (6 decimal digits) |
| Unused | Byte | 1 | 00-255 | Returns 0 |
| Unit Type | Byte | 1 | 00-255 | Code indicating unit type (30 +) |
| Unit Sub Type | Byte | 1 | 00-255 | Code indicating unit sub type |
| Unit ID Number | Byte | 1 | 00-255 | Code indicating unit ID number (1 to #units) |
| Config ID Number | Byte | 1 | 00-255 | Indicates ID code for loaded configuration (if applicable) |
| MP Rev | Byte | 1 | 00-255 | Indicates the MP Rev (MOD Level) used to build unit |
| Main SW Part# | Unsigned Double | 3 | 100000-999999 | Main Software Alto Part# (6 decimal digits). The MSB is not sent, it should be set to 0. |
| Unused | Byte | 3 |  | Return 0 |

Table of Manufacturing Information Data Fields

Operations Sent: Get

ManufacturingInformation Get (Host to Client):

| Class | Oper | Func | Seq | Length | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x01 | 0x70 | N | 0 | 26 bytes x 0x00 | 0xyy |

ManufacturingInformation Set (Host  to Client):  (Factory Only!!!)

| Class | Op | Func | Seq | Length | Data 1 | Data 2 | Data 3 | Data 4 | Data 5 | Data 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x00 | 0x70 | N | 22 | HW Ver | Reserved 1 | 0x00 | Reserved2 | 0x00 | Reserved3 |

| Data 7 | Data 8 | Data 9 | Data 10 | Data 11 | Data 12 | Data 13 | Data 14 | Data 15 | Data 16 | Data 17-22 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit S/N | Unit S/N | Unit S/N | Unit S/N | 0x00 | Amp Type | Amp Sub Type | Amp Number | Active Record | MP Rev | Ignored, read only | 4 bytes x 0x00 | 0xyy |

Operations Returned: Response, AckNak

ManufacturingInformation Response from Get (Client to Host):

| Class | Op | Func | Seq | Length | Data 1 | Data 2 | Data 3 | Data 4 | Data 5 | Data 6 | Data 7 | Data 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x06 | 0x70 | N | 26 | SW Ver | SW Ver | HW Ver | Reserved1 | 0x00 | Reserved2 | 0x00 | 0X00 |

| Data 9 | Data 10 | Data 11 | Data 12 | Data 13 | Data 14 | Data 15 |
| --- | --- | --- | --- | --- | --- | --- |
| 0x00 | Reserved3 | Unit S/N MSB | Unit S/N Byte 3 | Unit S/N Byte 2 | Unit S/N LSB | 0x00 |

| Data 16 | Data 17 | Data 18 | Data 19 | Data 20 | Data 21 | Data 22 | Data 23 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Amp Type | Amp Sub Type | Amp Number | Active Record | MP Rev | Main SW PN Byte3 | Main SW PN Byte2 | Main SW PN LSB |

| Data 24 | Data 25 | Data 26 | BCC |
| --- | --- | --- | --- |
| 0x00 | 0x00 | 0x00 | 0xyy |

ManufacturingInformation AckNak (Client to Host):

| Class | Oper | Func | Seq | Length | Data 1 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x07 | 0x70 | N | 1 | AckNak | 25 bytes x 0x00 | 0xyy |

      - PrepareToRestart (Class 0x01, Function 0x09)

PrepareToRestart and DiagRestart are paired messages of which the former allows a device to prepare for restart before receiving and executing the actual restart message. In the case of an Amplifier prepare means gracefully mute to avoid audio artifacts. In a Relay Controller, a discrete device, no application preparation is necessary. A watchdog timeout is induced to cause a warm restart. During this process an attempt is made to acknowledge that a DiagRestart is imminent.

Data Fields

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |

Table of PrepareToRestart Data Fields

Operations Sent: Set

PrepareToRestart Set (Host to Client):

| Class | Oper | Func | Seq | Length | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x00 | 0x09 | N | 0 | 26 bytes x 0x00 | 0xyy |

Operations Returned: AckNak

PrepareToRestart AckNak (Client to Host):

| Class | Oper | Func | Seq | Length | Data 1 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x07 | 0x70 | N | 1 | AckNak | 25 bytes x 0x00 | 0xyy |

      - DiagRestart (Class 0x01, Function 0x63)

Refer to the description of PrepareToRestart and DiagRestart in PrepareToRestart.

Data Fields

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
|  |  |  |  | Software Version |

Table of DiagRestart Data Fields

Operations Sent: MethodStart

DiagRestart Set (Host to Client):

| Class | Oper | Func | Seq | Length | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x80 | 0x63 | N | 0 | 26 bytes x 0x00 | 0xyy |

Operations Returned: AckNak

DiagRestart AckNak (Client to Host):

| Class | Oper | Func | Seq | Length | Data 1 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x07 | 0x70 | N | 1 | AckNak | 25 bytes x 0x00 | 0xyy |

### Clu Class 0x0B Functions

| Func | Name |
| --- | --- |
| 0x10 | CluInpInfo |
| 0x12 | CluInpOverride |
| 0x33 | CluMasterOff |

      - CluInpInfo (Class 0x0B, Function 0x10)

CluInpInfo is subject to significant change during development. Information available from the Config Table and Diagnostic Class Function DetailedDeviceStatus is not repeated here. Ultimately, all 26 available Data  Bytes will be used. Let's begin with just a few things then discover what else is really useful.

Debounce Data - when all all 8 samples are OFF the debounced state is OFF. Likewise with ON.   There is no m of n processing, for example, when 6 of 8 are OFF then the debounced state is OFF. External hardware like buttons and switches are expected to be stable after settling time.

Status Data - Bit 0 is the ON/OFF state adjusted for overrides thus not necessarily the pin state.

Bit 1 signals a change of debounced state from OFF->ON or ON->OFF occurred. Bit 2 is set during a hold delay. Bit 3 set indicates the algorithm is running. Bit 4  set indicates the algorithm associated with the Input is running but has temporarily paused its normal functionality. Alternative processing is active.

Parm 1 and Parm 2 Data reveal information pertinent to Sequence, Binary Count and Hot Cup algorithms. These three functions plus One Shot also use Parrm Config. Be careful to distinguish between Config/Cfg and Data/Dat. Parm Config is static after the algorithm is initialized unlike Parm 1 Data and Parm 2 Data which may or may not be dynamic while in the run state.

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Input Unit | Byte | 1 | 0 - 26 | The input unit's info requested |
| Debounce Data | Bit field | 1 | 8 single bits | Each bit is a sample, Bit 0 newest, 7 oldest. 1=ON, 0=OFF. |
| Status Data | Bit field | 1 | Bit 0: 1=on Bit 0: 1=change Bit 0: 1=hold Bit 0: 1=run Bit 0: 1=pause | 0=off 0=not change 0=not hold delay 0=not run 0=not pause |
| Parm 1 Data | Byte | 1 | 0 - 255 | Varies depending upon the Input function |
| Parm 2 Data | Byte | 1 | 0 - 255 | Varies depending upon the Input function |
| Parm 3 Data | Byte | 1 | 0 - 255 | Varies depending upon the Input function |

Table of CluInpInfo Data Fields

Operations Sent: Get

CluInpInfo Get (Host to Client):

| Class | Op | Func | Seq | Length | Data 1 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0B | 0x01 | 0x10 | N | 1 | Inp Unit | 25 bytes x 0x00 | 0xyy |

Operations Returned: Response, AckNak

CluInpInfo Data Response from Get (Client to Host):

| Class | Op | Func | Seq | Length | Data 1 | Data 2 | Data 3 | Data 4 | Data 5 | Data 6 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0B | 0x06 | 0x10 | N | 6 | Inp Unit | Debounce Data | Status Data | Parm 1 Data | Parm 2 Data | Parm 3 Data | 20 bytes x 0x00 | 0xyy |

CluInpInfo AckNak (Client to Host):

| Class | Oper | Func | Seq | Length | Data 1 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0B | 0x07 | 0x10 | N | 1 | AckNak | 25 bytes x 0x00 | 0xyy |

      - CluInpOverride (Class 0x0B, Function 0x12)

CluInpOverride provides an individually selectable override of the Pin State for each of the Inputs. There is an enable/disable and on/off bit for each of the inputs. With this capability input conditions can be simulated that would be difficult or impossible to duplicate with external hardware. This allows rigorous testing of individual algorithms and overall system control strategy. Race conditions, other intermittent relationships and error conditions can be easily introduced into the CLU. It is also a quick and easy way to debug fundamental logic errors.

Note that there are 27 Clu inputs thus the fourth of these bytes contains only three inputs.

Data Fields

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Inputs 0 - 7 Enable | Bit field | 1 | 8 one bit fields | 1=Enable Override, 0=Disable Override |
| Inputs 8 - 15 Enable | Bit field | 1 | 8 one bit fields | 1=Enable Override, 0=Disable Override |
| Inputs 16 - 23 Enable | Bit field | 1 | 8 one bit fields | 1=Enable Override, 0=Disable Override |
| Inputs 24 - 26 Enable | Bit field | 1 | 3 one bit fields | 1=Enable Override, 0=Disable Override |
| Inputs 0 - 7 OnOff | Bit field | 1 | 8 one bit fields | 1=ON, 0=OFF |
| Inputs 8 - 15 OnOff | Bit field | 1 | 8 one bit fields | 1=ON, 0=OFF |
| Inputs 16 - 23 OnOff | Bit field | 1 | 8 one bit fields | 1=ON, 0=OFF |
| Inputs 24 - 26 OnOff | Bit field | 1 | 3 one bit fields | 1=ON, 0=OFF |

Table of CluInpOverridel Data Fields

Operations Sent: Get, Set

CluInpOverride Get (Host to Client):

| Class | Op | Func | Seq | Length | Data 1 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0B | 0x01 | 0x12 | N | 1 | Inp Unit | 25 bytes x 0x00 | 0xyy |

CluInpOverride Set (Host to Client):

| Class | Oper | Func |  | Seq | Length |  | Data 1 |  | Data 2 |  | Data 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0B | 0x00 | 0x12 |  | N | 8 |  | Inputs 0 - 7 Enable |  | Inputs 8 - 15 Enable |  | Inputs 16 - 23 Enable |  |
| Data 4 |  |  | Data 5 |  |  | Data 6 |  | Data 7 |  | Data 8 |  | Padding |  | BCC |
| Inputs 24 - 26 Enable |  |  | Inputs 0 - 7 OnOff |  |  | Inputs 8 - 15 OnOff |  | Inputs 16 - 23 OnOff |  | Inputs 24 - 26 OnOff |  | 18 bytes x 0x00 |  | 0xyy |

CluInpOverride Returned: Response, AckNak

CluInpOverride Data Response from Get (Client to Host):

| Class | Oper | Func |  | Seq | Length |  | Data 1 |  | Data 2 |  | Data 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0B | 0x06 | 0x12 |  | N | 8 |  | Inputs 0 - 7 Enable |  | Inputs 8 - 15 Enable |  | Inputs 16 - 23 Enable |  |
| Data 4 |  |  | Data 6 |  |  | Data 7 |  | Data 8 |  | Data 9 |  | Padding |  | BCC |
| Inputs 24 - 26 Enable |  |  | Inputs 0 - 7 OnOff |  |  | Inputs 8 - 15 OnOff |  | Inputs 16 - 23 OnOff |  | Inputs 24 - 26 OnOff |  | 17 bytes x 0x00 |  | 0xyy |

CluInpOverride AckNak (Client to Host):

| Class | Oper | Func | Seq | Length | Data 1 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0B | 0x07 | 0x12 | N | 1 | AckNak | 25 bytes x 0x00 | 0xyy |

      - CluMasterOff (Class 0x0B, Function 0x33)

CluMasterOff resets all relays, indicators and algorithms. If this occurs while any input is active then after debouncing (< 50 ms) the input will turns its algorithm ON.

| Name | Data Type | Size in Bytes | Range | Description |
| --- | --- | --- | --- | --- |
| Dummy | Byte | 1 | 0xCF | Just a Dummy |

Table of CluMasterOff Control Data Fields

Operations Sent: Set

CluMasterOff Set (Host to Client):

| Class | Oper | Func | Seq | Len | Data 1 | Data Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0B | 0x00 | 0x33 | N | 1 | Dummy | 25 bytes x 0x00 | 0xyy |

CluMasterOff Returned: AckNak

CluMasterOff AckNak (Client to Host):

| Class | Oper | Func | Seq | Length | Data 1 | Padding | BCC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0B | 0x07 | 0x33 | N | 1 | AckNak | 25 bytes x 0x00 | 0xyy |

# Appendix

## AltoNET™ Interface Details

AltoNET™  supports RS-422 physical and data link layers and provides a proprietary applications layer message  protocol for communicating with Alto Products like Amplifiers and Relay Controllers.

### Physical Layer Details

The RS422 interface is differential with up to -10 to +10 Volt input range.

Differential signal should be at least +/- 2V fully loaded.

The protocol supports full duplex  and expects a baud rate of 115,200, 8 bits of data, no parity, 1 stop bit and no flow control.

The physical connections required implementing this interface as Inputs and Outputs are:

| Signal | PA700 BD | Main BD | Description |
| --- | --- | --- | --- |
| Tx+ | J5-27 | J2-9 | Transmit + |
| Tx- | J5-26 | J2-8 | Transmit - |
| Rx+ | J5-25 | J2-7 | Receive + |
| Rx- | J5-24 | J2-6 | Receive - |

### Message Format

All messages sent via AltoNET will have the same basic format shown below:

|  | Prefix | Class | Operation | Function | Sequence | Length | Data | BCC | CR/LF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Actual bits | 16 bits | 8 bits | 8 bits | 8 bits | 8 bits | 8 bits | <Data 1>…<Data N> | 8 bits | 16 bits |
| ASCII Xmitted bits | 32 bits | 16 bits | 16 bits | 16 bits | 16 bits | 16 bits | <Data 1>…<Data N> | 16 bits | 16 bits |

- The messages are sent and received as an ASCII string,  so ascii “AA550008” is actually sent and received as  0x41 0x41 0x35 0x35 0x30 0x30 0x30 0x38 ….

- ASCII string must be terminated with CR/LF  (0x0D 0x0A)

- The prefix is 0xAA 0x55

- The next Five bytes form the header.

- Unused bytes beyond the last byte of valid data are functionally ignored.

- BCC is calculated as a running XOR of each HEX byte in the message before conversion to ASCII, excluding the prefix (32 bytes).

- Message header, data length, and BCC are checked for validity in returned ACK/NACK.

- Each Data Field is 8 bits unless otherwise indicated throughout the document.

- There must be a minimum delay between sent messages of 50mS. //SJS confirm number***

The BCC Block Check Character  is an 8-bit Character created by performing consecutive exclusive OR (XOR) operation of the entire message byte-by-byte. The BCC is appended to the last byte of the message as showed in the next diagram.  It is always placed as the last byte (position 31).  Any unused bytes between the payload and BCC should be padded to 0x00

## Configuration Table Editing

Configuration table data Parameter Byte Values can be edited using the Alto Forte Manager.